Quantitative Continuous Score to Analyze HER2 expression in IHC slides [Document Under Preparation]

Introduction

Image segmentation for  cell and nuclei detection plays a crucial role in quantitative analysis of histopathology slides, particularly in the context of HER2 immunohistochemistry (IHC). Accurate detection and segmentation of cells and nuclei are foundational for assessing protein expression, cellular morphology, and ultimately, disease prognosis. This document outlines the methodologies and considerations for developing robust image segmentation techniques tailored for HER2 IHC stained slides.

Challenges in HER2 IHC Image Segmentation

HER2 IHC histopathology slides present unique challenges for image segmentation due to several factors:

- Staining Variability: Differences in laboratory protocols, reagent batches, and tissue processing can lead to variations in staining intensity and color.

- Cellular Heterogeneity: Tumor samples often contain a mix of different cell types, including tumor cells, stromal cells, and inflammatory cells, which can have diverse appearances.

- Overlapping and Clustered Cells: Cells in tissue sections can be densely packed and overlap, making it difficult to distinguish individual cell boundaries.

- Artifacts: Tissue folds, air bubbles, and debris can introduce noise and obscure cellular structures.

- Weak Staining: Some HER2 IHC cases may exhibit weak or heterogeneous staining, making it challenging to identify positively stained cells.

Segmentation Methodologies

Various image segmentation methodologies can be employed for cell and nuclei detection in HER2 IHC slides. These can broadly be categorized into traditional image processing techniques and deep learning approaches.

Traditional Image Processing Techniques

Traditional methods often rely on handcrafted features and mathematical operations to delineate structures.

- Thresholding: Global or adaptive thresholding can be used to separate foreground (stained cells/nuclei) from background. Otsu's method and adaptive Gaussian thresholding are common choices.

- Watershed Transform: This technique can be effective in separating touching or overlapping objects by treating the image as a topographic map. Markers (e.g., from nuclei detection) are often used to guide the segmentation.

- Morphological Operations: Erosion, dilation, opening, and closing operations can be applied to refine segmentation results, remove noise, and connect broken contours.

- Color Deconvolution: For IHC slides, color deconvolution can separate the stains (e.g., hematoxylin and DAB for HER2) into individual channels, simplifying subsequent segmentation steps.

Deep Learning Approaches

Deep learning models, particularly convolutional neural networks (CNNs), have demonstrated superior performance in image segmentation tasks.

- U-Net Architecture: The U-Net is a widely adopted architecture for biomedical image segmentation due to its ability to capture both local and global contextual information.

- Mask R-CNN: This instance segmentation model can simultaneously detect and segment individual cells and nuclei.

- Fully Convolutional Networks (FCNs): FCNs enable pixel-wise classification, making them suitable for semantic segmentation of cells and nuclei.

- Data Augmentation: To improve the robustness of deep learning models, data augmentation techniques (e.g., rotation, scaling, flipping, color jittering) are crucial to create a diverse training dataset.

- Transfer Learning: Pre-trained models on large natural image datasets can be fine-tuned on histopathology images, leveraging learned features and reducing the need for extensive training data.

Possible Workflow for Segmentation

A typical workflow for image segmentation in HER2 IHC histopathology slides would involve:

- Image Preprocessing:

  - Normalization of color and intensity to mitigate staining variability.

  - Noise reduction (e.g., median filtering).

  - Color deconvolution to separate hematoxylin (nuclei) and DAB (HER2 protein) stains.

- Nuclei Segmentation:

  - Applying a robust segmentation method (e.g., U-Net or thresholding followed by watershed) to identify individual nuclei.

  - Post-processing to refine nuclei boundaries and remove artifacts.

- Cell Segmentation:

  - Using the segmented nuclei as seeds or markers.

  - Employing region growing or another segmentation technique to expand from the nuclei to delineate the entire cell cytoplasm.

  - Consideration of HER2 positive and negative cell regions.

- Quantitative Analysis:

  - Counting of segmented cells and nuclei.

  - Measurement of features such as cell size, nuclear size, and HER2 staining intensity.

  - Classification of cells as HER2 positive or negative based on predetermined criteria.

Evaluation Metrics

The performance of segmentation algorithms can be evaluated using various metrics:

- Dice Coefficient (F1-score): Measures the similarity between the segmented output and the ground truth.

- Jaccard Index (IoU): Quantifies the overlap between the segmented region and the ground truth.

- Precision and Recall: Assess the accuracy of positive predictions and the ability to identify all relevant instances.

- Boundary F-measure: Evaluates the accuracy of boundary detection.

- Quantitative Scores: Clinical relevance metrics like HER2 score concordance.

Segmentation Approaches Tried:

In last 3 months following approaches have been tried for nuclei and cell membrane segmentation:

- Semantic Segmentation to select tumor region using hovernet

- Thresholding (OpenCV)

- Cellpose

- Stardist

- Combination of cellulose and stardist

- Unseg

- Segment anything model (SAM)

- Own approach with python code

Semantic Segmentation to select tumor region using hovernet

First, the hoevernet was tried for semantic segmentation (to select just the tumor region). Corresponding result is shown below, which show that hovernet (trained on H&E stained slides) is not working great for IHC stained slides for semantic segmentation: (https://github.com/Dizzaroo-Dev-Team/-digital_pathology_qcs/blob/main/ss_ihc.py )

Thresholding: In this method a color threshold is applied to select regions with blue and brown color. (Code: https://github.com/Dizzaroo-Dev-Team/-digital_pathology_qcs/blob/main/hsl_cmask_segmentation.py) An example of such segmentation is shown below

In this approach a contour was applied using opencv package to select regions containing nuclei and cell membrane. Further this selection was improved by (a) imposing restriction on area of contours to remove very small size nonspecific contours (b) selecting only those nuclei and membrane contours whose center coincides with each other.  (https://github.com/Dizzaroo-Dev-Team/-digital_pathology_qcs/blob/main/cmask_mn_janalysis.py).

The following result shows improved segmentation.

Cellpose: In this approach Cellpose code is used for both cell membrane and nuclei segmentation

Cellpose for nuclei segmentation on hematoxylin channel:

Stardist: In this approach Stardist code is used for nuclei segmentation. Stardist is just giving convex shaped nuclei and not able to select irregular shaped nuclei, therefore Stardist was not used further.

Stardist for nuclei segmentation on DAB channel:

Combination of cellpose and stardist:

Using cellpose for membrane and stardist for nuclei detection:

Stardist was used on hematoxylin channel and cellpose was used on DAB channel. Stardist is just giving convex shaped nuclei and not able to select irregular shaped nuclei, therefore Stardist was not used further.

Using cellpose for both membrane and nuclei detection:

Cellpose is able to do irregular selection, however using cellpose on different channels (hematoxylin and DAB) results in same membrane and nuclei membrane for tumor nuclei. Also such approach might only work with 3+ cases.

Unseg: In this approach probability based Unseg code is used for segmentation. This code is creating many unnecessary segmentations as shown below, therefore not used further.

If a successful semantic segmentation approach is available, then this might work as shown below:

Segment Anything Model: In this

Own Approach with Python Code:

Approach1:

- Take a patch

- Divide DAB and Hematoxylin channels

- On Hematoxylin channel remove the pixels where ODDAB > ODHematoxylin

- On DAB channel remove the pixels where  ODHematoxylin > ODDAB

- Find nuclei on hematoxylin channels

- Detect cell membrane around nuclei Based on

- Absence of anything: 3 micron distance

- Presence of other nuclei membrane

- Presence of DAB intensity

Approach1:

Approach1:

Details of algorithm and structure of code for QCS analysis

Files

- dslide.py

  - Tools to  work with WSI

- dpixel.py

  - Tools to analyse each pixel

- dseg.py

  - Tools for segmentation

- danalysis.py

  - Tools for analysis of patch and WSI

Details of Individual Files

- dslide.py: This file have functions to analyze whole slide images. It is structured as following :

  - Class Slide

    - print_slide_info(self)

    - get_thumbnail(self, size=(300,300), fname="thumbnail.png")

    - extract_patches_from_ndpi(self, output_dir='patches', patch_size=512, level=0)

  - filter_patches(patch_dir, filter_dir):

    - Works based on OD selection on H channel

    - Working but very slow

  - get_patches_clam(to write)

  - stain_normalization_patch(to write):

  - normalize_wsi(to write):

    - To normalize color and staining for standard analysis

- dseg.py

  - cellpose_seg(img_rgb, anno_fig='out.png', anno_file='out_cp.txt', anno_geojson='out.geojson', center_file='out_center.txt')

  - mem_approximate (to add)

    - Own code which will approximate membrane around nucleus

  - seeded_watershed_seg (to write)

  - clean_cp_nuclei_seg (to add)

  - plot_OD(img_file,den_file)

  - get_OD(img_rgb)

  - get_hed(img_file, h_file=None, e_file=None, d_file=None)

  - get_hd_clean(img_file, h_file=None, d_file=None)

  - get_cytoplasm (to add)

- dpixel.py

  - pixel_OD(img,i,j)

  - interpolate_point(A, B, r):

  - points_inside_polygon(image_shape, polygon_points):

  - intensity_weighted_center(image, polygon_points):

  - draw_tn(curve, length=5): #To get tangent and normals at points on the curve

  - points_in_polygon(polygon, shape):

  - pixels_between_curves(inner_curve, outer_curve, shape):

  - line_pixels(p1, p2):

- danalysis.py

  - get_mem_od

  - ppositive_cells

  - density_positive_cells

  - get_NMR

- xx

Examples of dqcs package use

xx